# Accessible Radar Workstation – Developer Handoff

# 01 Product Requirements Document

# Accessible Radar Workstation (ARW) – Product Requirements Document

## Vision

A standalone Windows application enabling blind users to analyze weather radar data through keyboard navigation,

speech output, and spatialized audio.

## Primary Capabilities

- Radar site switching

- Storm object detection

- Motion tracking

- Radial velocity analysis

- Hail detection

- Debris signature detection

- Spatial audio scene representation

## Target Users

Blind weather enthusiasts and analysts.

## Success Criteria

Users must be able to:

1. Switch radar sites

2. Identify precipitation objects

3. Hear storm motion

4. Detect hail likelihood

5. Detect debris signatures

6. Operate entirely with keyboard and speech feedback

# 02 Technical Architecture

# Technical Architecture

High level pipeline:

Weather Sources

↓

Ingest Manager

↓

Cache Store

↓

Product Parsers

↓

Object Extraction Engine

↓

Tracking Engine

↓

Hazard Analysis Engine

↓

Scene Model

↓

Speech Manager + Audio Renderer

↓

NVGT UI

Key principle: only the ingest manager makes network calls.

# 03 Developer Task Breakdown

# Developer Task Breakdown

## Phase 1

- Site database

- Reflectivity ingest

- Rain object detection

- Speech summaries

## Phase 2

- Motion tracking

- Replay buffer

## Phase 3

- Velocity ingestion

- Velocity region detection

## Phase 4

- Hail detection

- Debris scoring

## Phase 5

- Spatial audio scene

# 04 Data Source Specification

# Data Source Specification

Primary source:

NEXRAD Level II radar data

Fields:

- Reflectivity

- Velocity

- Spectrum width

- Correlation coefficient

- Differential reflectivity

Access:

AWS Open Data

Bucket:

s3://noaa-nexrad-level2/

Typical scan cadence:

4–6 minutes precipitation mode

# 05 Object Detection Algorithms

# Object Detection Algorithms

Reflectivity thresholds:

5–20 dBZ: drizzle

20–30 dBZ: light rain

30–40 dBZ: moderate rain

40–50 dBZ: heavy rain

50–60 dBZ: intense rain

60+ dBZ: severe core

Velocity regions:

negative velocity → inbound

positive velocity → outbound

Rotation detection:

strong inbound adjacent to strong outbound

Debris scoring inputs:

- high reflectivity

- rotation

- low correlation coefficient

# 06 Radar Site Selection Beam Height

# Radar Site Selection and Beam Height Algorithm

Goal:

Select the radar that observes a storm at the lowest useful beam height.

## Beam Height Formula

beam_height =

distance * tan(elevation_angle) +

(distance² / (2 * earth_radius)) +

radar_elevation

Typical values:

lowest elevation angle ≈ 0.5°

earth radius ≈ 6371 km

## Algorithm

For each radar site:

1. compute distance to storm target

2. compute beam height at target

3. reject if beam height too high (>10 km)

4. rank remaining radars by lowest beam height

Best radar = lowest beam height over storm.

## Pseudocode

for radar in radars:

distance = calc_distance(radar, target)

beam_height = calc_beam_height(distance)

if beam_height < threshold:

candidates.append(radar)

best_radar = min(candidates, key=beam_height)